Specifications

Contents

General	3

General Radio Specification	3

Receiver	3

Transmitter	3

Gallery	5

# General

## General Radio Specification

| Name | RT24 |
| --- | --- |
| Brand | Retevis |
| Channels | 16 |
| Frequency Range | 149.0250-149.1125MHz PMR 446 |
| Battery |  |
| Working Voltage | 3.7 DC |
| Effective Radiated Power | Low: 0.5W High: 2.0W |
| Operating Temperature | -10oC to +50oC |
| Antenna | Fixed |
| Antenna Impedance | 50Ω |
| Scan | Yes |
| Vox | Yes |
| Battery Save | Yes |
| Bandwidth | 12.5Khz |
| CTCSS/DCS | Yes |
| Low Battery Alarm | Yes |
| Busy Channel Lock | Yes |
| Working Mode | Simplex |
| Time-out-Time (TOT) | 180s |
| Voice | English Chinese |
| Squelch Adjustment | Yes |
| Weight (Including Battery) | 193g |
| PC Program Protection | Yes |
| Dimensions | 230*62*34mm |

## Receiver

| RF Sensitivity | ≤0.2µV |
| --- | --- |
| Occupied Bandwidth | ≤16KHz |
| Selectivity | ≥65dB |
| Intermediation | ≥55dB |
| Audio Power | >500mW |
| Audio Distortion | ≤5% |
| Frequency Stability | 5ppm |
| Current | Standby: 55mA Working: 150mA |
| Audio Response | (300-3000Hz):+7~-12.5dB |

## Transmitter

| Modulation Type | 16K¢F3F |
| --- | --- |
| Spurious Radiation | ≤7.5µW |
| Modulated Noise | <-40dB |
| Modulation Distortion | <5% |
| Frequency Stability | 5ppm |
| Max. Fr Deviation | ≤±5KHz |
| Current | ≤1200mA |
| Audio Response | (300-3000Hz):+6.5~-14dB |
| Adjacent CH. Power | ≥65dB |
| Intermediation Sensitivity | 8-12mv |

# Gallery

| Description | Image |
| --- | --- |
| Front |  |